Packet Tracer - Configuring Named Standard ACLs (Instructor Version)

Instructor Note: Red font color or Gray highlights indicate text that appears in the instructor copy only.

## Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | F0/0 | 192.168.10.1 | 255.255.255.0 | N/A |
|  | F0/1 | 192.168.20.1 | 255.255.255.0 | N/A |
|  | E0/0/0 | 192.168.100.1 | 255.255.255.0 | N/A |
|  | E0/1/0 | 192.168.200.1 | 255.255.255.0 | N/A |
| File Server | NIC | 192.168.200.100 | 255.255.255.0 | 192.168.200.1 |
| Web Server | NIC | 192.168.100.100 | 255.255.255.0 | 192.168.100.1 |
| PC0 | NIC | 192.168.20.3 | 255.255.255.0 | 192.168.20.1 |
| PC1 | NIC | 192.168.20.4 | 255.255.255.0 | 192.168.20.1 |
| PC2 | NIC | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 |

Objectives

Part 1: Configure and Apply a Named Standard ACL Part 2: Verify the ACL Implementation

Background / Scenario

The senior network administrator has tasked you to create a standard named ACL to prevent access to a file server. All clients from one network and one specific workstation from a different network should be denied access.

Packet Tracer - Configuring Named Standard ACLs

Part 1:	Configure and Apply a Named Standard ACL

Step 1:	Verify connectivity before the ACL is configured and applied.

All three workstations should be able to ping both the Web Server and File Server.

Step 2:	Configure a named standard ACL.

Configure the following named ACL on R1.

R1(config)# ip access-list standard File_Server_Restrictions

R1(config-std-nacl)# permit host 192.168.20.4

R1(config-std-nacl)# deny any

Note: For scoring purposes, the ACL name is case-sensitive.

### Step 3:	Apply the named ACL.

- Apply the ACL outbound on the interface Fast Ethernet 0/1.

R1(config-if)# ip access-group File_Server_Restrictions out

- Save the configuration.

# Part 2:	Verify the ACL Implementation

### Step 1:	Verify the ACL configuration and application to the interface.

Use the show access-lists command to verify the ACL configuration. Use the show run or show ip interface fastethernet 0/1 command to verify that the ACL is applied correctly to the interface.

### Step 2:	Verify that the ACL is working properly.

All three workstations should be able to ping the Web Server, but only PC1 should be able to ping the File Server.